WORKING CODE:

#include <WiFi.h>

#include <HTTPClient.h>

#define RXD2 16

#define TXD2 17

const char* ssid = "OnePlus Nord CE5 z7ce";

const char* password = "wjac2877";

String apiKey = "VKEHGV4CW6FUS4YK";

uint8_t npkQuery[]  = {0x01,0x03,0x00,0x1E,0x00,0x03,0x65,0xCD};

uint8_t phQuery[]   = {0x01,0x03,0x00,0x06,0x00,0x01,0x64,0x0B};

uint8_t moistQuery[]= {0x01,0x03,0x00,0x12,0x00,0x01,0x24,0x0F};

uint8_t tempQuery[] = {0x01,0x03,0x00,0x13,0x00,0x01,0x75,0xCF};

uint8_t condQuery[] = {0x01,0x03,0x00,0x15,0x00,0x01,0x95,0xCE};

uint8_t resp[11];

void sendQuery(uint8_t *q) {

while (Serial2.available()) Serial2.read();

Serial2.write(q, 8);

delay(300);

for(int i = 0; i < 11; i++){

if(Serial2.available()){

resp[i] = Serial2.read();

} else {

resp[i] = 0;

}

}

}

void connectWiFi() {

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

}

void setup(){

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, RXD2, TXD2);

connectWiFi();

}

void loop(){

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Reconnecting WiFi...");

connectWiFi();

}

sendQuery(moistQuery);

delay(100);

float moisture = (resp[3]<<8 | resp[4]) / 10.0;

sendQuery(tempQuery);

delay(100);

float temp = (resp[3]<<8 | resp[4]) / 10.0;

sendQuery(phQuery);

delay(100);

float ph = (resp[3]<<8 | resp[4]) / 100.0;

sendQuery(condQuery);

delay(100);

int ec = (resp[3]<<8 | resp[4]);

sendQuery(npkQuery);

delay(100);

int n = (resp[3]<<8 | resp[4]);

int p = (resp[5]<<8 | resp[6]);

int k = (resp[7]<<8 | resp[8]);

Serial.println("----- SENSOR DATA -----");

Serial.print("Moisture: "); Serial.println(moisture);

Serial.print("Temp: "); Serial.println(temp);

Serial.print("pH: "); Serial.println(ph);

Serial.print("EC: "); Serial.println(ec);

Serial.print("N: "); Serial.println(n);

Serial.print("P: "); Serial.println(p);

Serial.print("K: "); Serial.println(k);

if(WiFi.status() == WL_CONNECTED){

HTTPClient http;

String url = "http://api.thingspeak.com/update?api_key=" + apiKey +

"&field1=" + String(moisture) +

"&field2=" + String(temp) +

"&field3=" + String(ph) +

"&field4=" + String(ec) +

"&field5=" + String(n) +

"&field6=" + String(p) +

"&field7=" + String(k);

Serial.println("Sending to ThingSpeak...");

Serial.println(url);

http.begin(url);

int httpCode = http.GET();

Serial.print("HTTP Response: ");

Serial.println(httpCode);

if(httpCode == 200){

Serial.println("Data uploaded successfully!");

} else {

Serial.println(" Upload failed!");

}

http.end();

}

Serial.println("-----------------------------");

delay(20000);

}

PARTIALLY WORKING CODE:

#include <WiFi.h>

#include <HTTPClient.h>

const char* ssid = "OnePlus Nord CE5 z7ce";

const char* password = "wjac2877";

String apiKey = "VKEHGV4CW6FUS4YK";

#define RXD2 16

#define TXD2 17

uint8_t npkQuery[]   = {0x01,0x03,0x00,0x1E,0x00,0x03,0x65,0xCD};

uint8_t phQuery[]    = {0x01,0x03,0x00,0x06,0x00,0x01,0x64,0x0B};

uint8_t moistQuery[] = {0x01,0x03,0x00,0x12,0x00,0x01,0x24,0x0F};

uint8_t tempQuery[]  = {0x01,0x03,0x00,0x13,0x00,0x01,0x75,0xCF};

uint8_t condQuery[]  = {0x01,0x03,0x00,0x15,0x00,0x01,0x95,0xCE};

uint8_t resp[11];

bool sendQuery(uint8_t *q) {

while (Serial2.available()) Serial2.read();

Serial2.write(q, 8);

delay(300);

int i = 0;

unsigned long start = millis();

while (i < 11 && millis() - start < 1000) {

if (Serial2.available()) {

resp[i++] = Serial2.read();

}

}

if (i < 7) return false;

return true;

}

void connectWiFi() {

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\n WiFi Connected!");

}

void sendToThingSpeak(float moisture, float temp, float ph, int ec, int n, int p, int k) {

if (WiFi.status() != WL_CONNECTED) return;

HTTPClient http;

String url = "http://api.thingspeak.com/update?api_key=" + apiKey +

"&field1=" + String(moisture) +

"&field2=" + String(temp) +

"&field3=" + String(ph) +

"&field4=" + String(ec) +

"&field5=" + String(n) +

"&field6=" + String(p) +

"&field7=" + String(k);

Serial.println("📡 Sending to ThingSpeak...");

Serial.println(url);

http.begin(url);

int httpCode = http.GET();

Serial.print("HTTP Response: ");

Serial.println(httpCode);

if (httpCode == 200) {

Serial.println(" Upload Success");

} else {

Serial.println(" Upload Failed");

}

http.end();

}

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, RXD2, TXD2);

connectWiFi();

}

void loop() {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Reconnecting WiFi...");

connectWiFi();

}

float moisture = 0, temp = 0, ph = 0;

int ec = 0, n = 0, p = 0, k = 0;

if (sendQuery(moistQuery)) {

moisture = (resp[3] << 8 | resp[4]) / 10.0;

}

if (sendQuery(tempQuery)) {

temp = (resp[3] << 8 | resp[4]) / 10.0;

}

if (sendQuery(phQuery)) {

ph = (resp[3] << 8 | resp[4]) / 100.0;

}

if (sendQuery(condQuery)) {

ec = (resp[3] << 8 | resp[4]);

}

if (sendQuery(npkQuery)) {

n = (resp[3] << 8 | resp[4]);

p = (resp[5] << 8 | resp[6]);

k = (resp[7] << 8 | resp[8]);

}

Serial.println("\n----- SENSOR DATA -----");

Serial.print("Moisture: "); Serial.println(moisture);

Serial.print("Temp: "); Serial.println(temp);

Serial.print("pH: "); Serial.println(ph);

Serial.print("EC: "); Serial.println(ec);

Serial.print("N: "); Serial.println(n);

Serial.print("P: "); Serial.println(p);

Serial.print("K: "); Serial.println(k);

Serial.println("----------------------");

sendToThingSpeak(moisture, temp, ph, ec, n, p, k);

delay(20000);

}